Regulations concerning lifesaving appliances on cargo ships – Draft

Laid down by the Norwegian Maritime Directorate on (date) pursuant to the Act no. 7 of 9 June 1903 relating to Public Control of the Seaworthiness of Ships, etc., sections 1, 41a, 42, 47, 48, 97 and 100, cf. Royal Decree no. 9 of 5 April 1963 and Royal Decree no. 12 of 1 December 1978.

Section 1 Scope of application	1

Section 2 Definitions	2

Section 3 Obligations of the company and the master	3

Section 4 Exemptions	4

Section 5 Documentation	4

Section 6 Requirements for approval of equipment, etc.	4

Section 7 Lifesaving appliances	5

Section 8 Survival craft	5

Section 9 Hydrostatic release unit	6

Section 10 Requirements for stowing survival craft – embarkation and launching appliances	6

Section 11 Rescue boats	6

Section 12 Embarkation and launching appliances for rescue boats	7

Section 13 Muster stations	7

Section 14 Lifebuoys	7

Section 15 Hand flares/Rocket-parachute flares/Line-throwing appliances	8

Section 16 Lifejackets	8

Section 17 Immersion suits	8

Section 18 Labelling of storage locations	8

Section 19 Operating instructions	8

Section 20 General alarm system, etc.	9

Section 21 Muster list	9

Section 22 Manning and command of survival craft	10

Section 23 Training	10

Section 24 Drills	10

Section 25 Operational readiness, maintenance and inspections	11

Section 26 Penalty	11

Section 27 Entry into force	11

Annex 1 (cf. section 10) Requirements for stowing survival craft – embarkation and launching appliances	12

Annex 2 (cf. section 13) Muster stations	14

Annex 3 (cf. section 20) General alarm and loudspeaker system	15

Annex 4 (cf. section 23) Onboard training and instruction	16

Annex 5 (cf. section 24) Drills	17

Annex 6 (cf. section 25) Operational readiness, maintenance and inspections	19

# Section 1

# Scope of application

(1) These regulations shall apply to all cargo ships and manned lighters irrespective of size, trade area and construction date.

(2) Cargo ships in great coasting or larger trade areas, plus cargo ships with a gross tonnage of 500 and upwards irrespective of trade area, shall comply with all of the requirements for lifesaving equipment specified in chapter III of the International Convention for the Safety of Life at Sea (SOLAS 1974) that are in force at any given time. In addition, sections 2 to 7, sections 11, 17 and sections 26 and 27 of these regulations will apply.

# Section 2

# Definitions

For the purpose of this Regulation, the following definitions shall apply:

- Recognized classification society: classification societies with which the Ministry has entered into an agreement pursuant to section 9 of the Seaworthiness Act:

  - Det Norske Veritas (DNV).

  - Lloyd's Register of Shipping (LRS).

  - Bureau Veritas (BV).

  - Germanischer Lloyd (GL).

  - American Bureau of Shipping (ABS).

b)	Accepted:  Equipment accepted by the Norwegian Maritime Directorate based on approval or type-approval of the equipment by a recognized classification society, another public or private institution, or the administration in a country that has ratified the SOLAS Convention.

c)	Manned lighter: A lighter for which the manning has been specified pursuant to the regulations relating to the manning of Norwegian ships that are in force at any given time.

d)	Gross tonnage: The number entered as gross tonnage in the Tonnage Certificate. If safety tonnage is entered in the remarks column of the Tonnage Certificate, this tonnage shall be used as gross tonnage.

e)	Built:

- That the ship is at a stage of construction where the keel is laid; or

- the construction of a specific ship can be established as having commenced; or

- the parts of the ship that have been put together comprise at least 50 tons or constitute 1% of the calculated weight of the building material, whichever is less.

f)	Float-free launching: The method of launching survival craft, where the craft is automatically released from the sinking ship and is ready for use.

g)	Free-fall launching: The method of launching survival craft, where the craft with persons and equipment on board is released and allowed to fall into the sea without any form of device that can restrain its fall.

h)	IMO (International Maritime Organisation): The International Maritime Organisation

i)	Embarkation ladders: Ladders stowed at embarkation stations for survival craft to facilitate safe access to the survival craft after launching

j)	Length (L): 96 per cent of the total length on a waterline at 85 per cent of the least moulded depth measured from the top of the keel, or the length from the fore side of the stem to the axis of the rudder stock on that waterline, whichever is the greater. In ships designed with a rake of keel, the waterline on which this length is measured shall be parallel to the designed waterline.

k)	LSA Code (International Lifesaving Appliance Code): The International Lifesaving Appliance (LSA) Code, adopted by the Maritime Safety Committee of the IMO by resolution MSC.48(66), as amended by the IMO.

l)	Cargo ship: Any ship that is not a passenger ship, fishing vessel, lighter or pleasure craft. In these regulations, the following definitions apply:

1.	Passenger ship: A ship for which a passenger certificate or safety certificate for passenger ships is required under the provisions of Chapter VIII of Act no. 7 of 9 June 1903 relating to Public Control of the Seaworthiness of Ships, etc.

2.	Fishing vessel: Any vessel used commercially for catching fish, including whales, seals, seaweed and sea tangle or other living resources of the sea.

3.	Pleasure craft: Any floating device intended for and capable of movement on water and not used for commercial purposes.

4.	Lighter: A hull or ship without propulsion machinery which is towed or pushed for all moves, and which is used for carrying cargo.

m)	Rescue boat: A boat that is designed to save persons in distress and to marshal survival craft.

n)	Immersion suit: A protective suit that reduces loss of body heat for a person immersed in cold water. It must be insulated.

o)	Survival craft: A lifeboat, rescue boat, fast rescue boat, or life raft.

p)	The International Convention for the Safety of Life at Sea (SOLAS): The International Convention of 1974 for the Safety of Life at Sea, as amended by the IMO.

q)	Tanker: A cargo ship built or outfitted for the carriage of flammable liquid cargo in bulk, including chemicals, oil and natural gas.

r)	Thermal protective equipment: A pouch or suit made of waterproof material with a low thermal conductivity.

s)	Launching device or launching appliance: A device that can safely transfer a survival craft or rescue boat from its stowed position into the water.

# Section 3

# Obligations of the company and the master

The company and the master shall ensure compliance with the provisions of these regulations.

# Section 4

# Exemptions

The Norwegian Maritime Directorate may, in individual cases and upon written application, grant exemption from the requirements of these regulations. There must be special reasons that make the exemption necessary, and it must be justifiable in terms of safety. Exemptions can only be granted where they do not contravene international agreements to which Norway has acceded.

# Section 5

# Documentation

Documentation shall be submitted to the Norwegian Maritime Directorate in accordance with the valid list of drawings prepared by the Norwegian Maritime Directorate at any given time.

# Section 6

# Requirements for approval of equipment, etc.

(1) Equipment that is required pursuant to these regulations, that is covered by regulations no. 1455 of 29 December 1998 concerning Marine Equipment (the Marine Equipment Regulations) and that is brought on board after these regulations have entered into force, shall be approved by a technical control body pursuant to the same regulations and be wheel-marked.

(2) Equipment that is not covered by the Marine Equipment Regulations shall be accepted.

(3) Previously approved or accepted equipment that was brought on board before these regulations entered into force can still be used until the equipment is replaced.

(4) Equipment that is not required pursuant to these regulations, but that is voluntarily installed onboard, shall meet the requirements of this section.

(5) In connection with significant reconstruction or repair of the ship that will have an effect on lifesaving equipment, the Norwegian Maritime Directorate can decide that new equipment shall be installed onboard. The same applies to any alteration in the ship’s operation pattern and manning or extension of trade area.

(6) If after a demonstration of the rescue appliance, the Norwegian Maritime Directorate finds that it does not function as intended, or that it can put the life and health of passengers and crew in danger on the basis of the operation and manning of the ship, the Norwegian Maritime Directorate can demand that the equipment be replaced or the appliance be altered.

# Section 7

# Lifesaving appliances

Depending on the trade area and tonnage, every ship with a gross tonnage of less than 500 shall at least carry the lifesaving appliances specified in the table below.

| Trade area/tonnage Equipment | All ships in trade areas 1 and 2 | All ships in trade area 3 and larger | Manned lighters | Section |
| --- | --- | --- | --- | --- |
| Survival craft | 200% | 200% | 100% | Section 8 |
| Rescue boat Gross tonnage under 100 Gross tonnage over 100 | - 1 | - 1 | - | Section 11 |
| Lifebuoys Gross tonnage under 300 Gross tonnage over 300 | 3 5 | 3 5 | 3 5 | Section 14 |
| Lifejackets | 150% | 150% | 150% | Section 16 |
| Lifejackets for children | For all children on board | For all children on board | - | Section 16 |
| Immersion suits | 100% | 100% | 100% | Section 17 |
| Rocket parachute flares | 3 | 6 | - | Section 15 |
| Hand flares | 3 | 6 | - | Section 15 |
| Line-throwing appliance Gross tonnage under 100 Gross tonnage over 100 | 0 1 | 1 1 | - | Section 15 |

# Section 8

# Survival craft

(1) With the exception of tankers, all ships shall carry at least one survival craft on each side of the ship, and that has sufficient capacity to at least hold the total number of passengers and crew. If only one life raft is carried in trade areas 1 and 2, it must be possible to launch this life raft from either side of the ship.

(2) There shall be a sufficient number of survival craft on board so that if a survival craft wrecks or becomes unusable the remaining survival craft will still have the capacity to hold the total number of persons that the ship is certified to carry.

(3) Tankers shall either carry a canopied lifeboat on each side of the ship or a free-fall lifeboat located astern with sufficient capacity to hold at least the total number of passengers and crew.

(4) Manned lighters in trade area 3 or a larger area shall be equipped with at least one inflatable life raft with sufficient capacity to hold everyone on board. Manned lighters in trade areas 1 and 2 shall be equipped with a CE-labelled motor-powered workboat that at least meets the requirements of design category C, or a life raft that can at least hold the total number of passengers and crew. For manned lighters carrying a flammable liquid cargo, the Norwegian Maritime Directorate sets requirements for each individual case, depending on the lighter’s manning, trade area, design and operational conditions.

# Section 9

# Hydrostatic release unit

All life rafts shall be equipped with a hydrostatic release unit and be stowed in a place where they can float free, cf. annex 1.

# Section 10

# Requirements for stowing survival craft – embarkation and launching appliances

Survival craft shall be stowed in accordance with annex 1.

# Section 11

# Rescue boats

(1) Ships built on 1 January 2005 or later with a gross tonnage of 100 and upwards shall carry a rescue boat.

(2) Irrespective of the construction date, ships with a gross tonnage of less than 100 do not need to carry a rescue boat if:

a)	The ship is equipped with a CE-labelled motor-powered workboat that at least meets the requirements of design category C, or

b)	The ship has the manoeuvrability under any condition to get close to and in a simple way take on board persons from the sea.

(3) Ships built between 1 July 1986 and 1 January 2005 with a gross tonnage above 500 shall carry a rescue boat.

(4) Ships with a gross tonnage between 100 and 500 shall, irrespective of trade area, be equipped with a rescue boat by the first initial survey or intermediate survey after 1 January 2006, whichever comes first.

(5) The requirement for a rescue boat can be met by having a lifeboat that meets the requirements for a rescue boat.

(6) The rescue boat can be included in the capacity of survival craft as mentioned in Section 8 in trade areas 1 and 2.

(7) The rescue boat and CE-labelled workboat shall be actuated by their own launching appliances, which can handle both launching and recovery while the ship is moving forward.

# Section 12

# Embarkation and launching appliances for rescue boats

(1) Embarkation and launching appliances for rescue boats shall be such that the boat can be boarded and launched in as little time as possible. The maximum time to complete this operation shall not exceed 5 minutes.

(2) It must be possible for the number of persons who are designated to be crew in the rescue boat to board and launch the rescue boat directly from its stowed position.

(3) The launching appliance shall meet the requirements in annex 1. It must be possible to launch the rescue boats while the ship is moving forward at a speed of at least 5 knots.

(4) It must be possible to recover a rescue boat in no more than five minutes when it is fully loaded with persons and equipment.

(5) It must be possible to observe the launching and recovery from the bridge.

# Section 13

# Muster stations

Launching stations shall be in accordance with the specifications in annex 2.

# Section 14

# Lifebuoys

(1) At least one of the lifebuoys on each side shall be equipped with a buoyant lifeline with a length of at least 30 metres.

(2) At least one of the lifebuoys on each side shall be equipped with a self-igniting smoke signal and self-igniting light, but without a line. It must be possible to release them quickly from the bridge.

(3) The remaining lifebuoys shall be equipped with self-igniting lights, but without a line.

(4) For vessels that shall only have 3 lifebuoys, one of the lifebuoys shall be equipped with a lifeline at least 30 metres in length, and one of the lifebuoys shall have a self-igniting smoke signal and self-igniting light.

(5) I special cases, the Norwegian Maritime Directorate can set other requirements for lifebuoys besides those specified in sub-sections 1 to 4 above, depending on the ship’s manning, trade area, design and operational conditions.

# Section 15

# Hand flares/Rocket-parachute flares/Line-throwing appliances

Hand flares, rocket-parachute flares, line-throwing appliances and other safety equipment that are not fixed, meant for emergency situations or shall be carried on board the lifesaving appliances under evacuation shall be stored in or in the direct vicinity of the wheelhouse.

# Section 16

# Lifejackets

(1) There shall be enough lifejackets for everyone on board. In addition, there shall be lifejackets for at least another 50% of the total number of passengers and crew.

(2) Lifejackets shall be stored in places that are easily accessible on board. They shall be stored in clearly marked and well-ventilated boxes or cabinets near the muster or embarkation stations and in places where personnel on watches may usually be found.

(3) In addition, lifejackets shall be carried for all children on board. A person is considered a child when being between 1 year and 35 kilograms. Lifejackets for children shall be stored separately from lifejackets for adults and shall be easily accessible.

# Section 17

# Immersion suits

There shall be immersion suits with thermal insulation for everyone onboard all bulk carriers irrespective of trade area and for other cargo ships in trade areas outside 30˚ N and 30˚ S from 1 January 2006.

# Section 18

# Labelling of storage locations

Storage locations for lifesaving appliances shall be labelled with their content and the number of each appliance in a clear manner.

# Section 19

# Operating instructions

There shall be easily visible placard or sign on or in the vicinity of survival craft and launching appliances that in a clear manner shall:

a) illustrate and provide instructions for the operation of the launching appliance and             give information about any elements of danger, and

b)  be readable in emergency lighting.

# Section 20

# General alarm system, etc.

(1) Ships with a gross tonnage of 200 and upward shall be equipped with a general alarm system.

(2) The general alarm system shall meet the requirements in annex 3.

(3) The general alarm system shall be able to summon the passengers and crew to the muster stations and initiate the operations that are specified in the ship’s muster list.

(4) For ships with a gross tonnage of less than 200, an alarm signal given with the ship’s whistle or siren instead of a general alarm system may be accepted.

(5) The general alarm system shall be supplemented with:

a)	a loudspeaker system in accordance with the specifications in annex 3, or

b)	an approved rousing and calling-up installation, or

c)	other suitable means of communication.

(6) It must be possible for the communication specified in sub-section 5 to reach all rooms where the crew may usually be found.

# Section 21

# Muster list

(1) A muster list shall be drawn up.

(2) The list shall be posted in easily visible places onboard, including on the bridge and in the crew’s accommodations.

(3) The muster list shall be written in the working language that is used on board.

(4) The muster list shall give detailed information about the general alarm and communications system as mentioned in section 20 and shall indicate how the passengers and crew shall behave when an alarm is sounded. It shall also specify how the order to abandon ship will be given.

(5) The duties that are imposed on the different members of the crew shall be specified in the muster list, including:

a) closing of watertight doors, fire doors, portholes, drains, hatches, skylights, side scuttles and other similar openings on the ship;

b) responsibility for bringing along unattached lifesaving equipment;

c) readying and launching of survival craft;

d) general readying of other lifesaving appliances;

e) mustering

f) use of communications equipment

g) composition of the fire brigade that shall fight a fire; and

h) special duties in the use of fire-extinguishing appliances and installations.

(6) The muster list shall specify which officers are responsible for ensuring that the lifesaving and fire-extinguishing appliances is kept in good condition and is ready for immediate use.

(7) The substitutes for key personnel shall be specified on the muster list, where it is taken into consideration that different emergency situations may require different measures.

# Section 22

# Manning and command of survival craft

(1) There shall be a sufficient number of crew members on board to operate all the ship’s survival crafts and the launching appliances.

(2) The masters of rescue boats and the fast rescue boats must have certified training and a certificate pursuant to the regulations in force at any given time concerning qualification requirements and certificate rights for personnel on Norwegian ships.

(3) Persons who have been designated to take command of survival craft that are launched by means of a launching appliance shall have training and a certificate pursuant to the regulations in force at any given time concerning qualification requirements and certificate rights for personnel on Norwegian ships.

# Section 23

# Training

(1) All ships and manned lighters shall have a training manual, which is easily accessible to the crew.

(2) The training manual shall include instructions and information with illustrations, where possible, about the ship’s lifesaving appliances and fire equipment.

(3) Every member of the crew who shall perform duties in an emergency situation shall be trained in his/her duties before the ship’s departure.

(4) Onboard training in the use of the ship’s lifesaving appliances and fire equipment shall be given in accordance with annex 4.

(5) It shall be possible to document the training for each member of the crew.

# Section 24

# Drills

(1) Every crew member shall take part in at least one ship drill and one fire drill each month.

(2) If more than 25% of the crew are replaced in under one month, ship and fire drills shall be held no later than 24 hours after the ship has left port.

(3) When a ship is put into service for the first time after major renovations or when a new crew has been hired, drills shall be conducted prior to departure.

(4) Every lifeboat shall be launched and manoeuvred in the water at least once every third month.

(5) Rescue boats and fast rescue boats shall be launched and manoeuvred in the water at least once a month.

(6) It must be possible to document all drills.

(7) To the extent that it is practically possible, drills shall be conducted as if it were a real emergency situation.

(8) When conducting drills, annex 5 and the ship’s muster list shall be followed.

# Section 25

# Operational readiness, maintenance and inspections

(1) As long as the vessel is operative, all lifesaving appliances shall comply with applicable regulations and be ready for immediate use.

(2) The provisions in annex 6 relating to maintenance of lifesaving equipment must be complied with.

(3) It must be possible to document the maintenance.

# Section 26

# Penalty

Wilful or negligent violation of these regulations is punishable by fines under section 339, subsection 2 of Act no. 10 of 22 May 1902 (the General Civil Penal Code), cf. section 48a and section 48b, unless a more severe penalty is applicable under another statutory provision.

# Section 27

# Entry into force

(1) These regulations enter into force on 1 January 2005.

(2) At the same time, the regulations no. 700 of 15 September 1992 concerning Lifesaving Appliances, etc. on Passenger Ships and Cargo Ships will be repealed.

# Annex 1 (cf. section 10)

# Requirements for stowing survival craft – embarkation and launching appliances

1. Each survival craft shall be stowed:

1.1	so that neither the survival craft nor the appliances complicate the operation of any other survival craft or rescue boat at any other launching station;

1.2	as close to the surface of the water as it is safely and practically possible to get and, with regard to survival craft other than throw-overboard life rafts, so that the survival craft in the embarkation position is at least 2 metres above the water line with the ship in fully-loaded condition under unfavourable trim conditions of up to 10° and with 20° listing to either of the sides or at the angle where the edge of the weather deck comes under water, whichever occurs first;

1.3	in a condition of permanent readiness so that the crew members can ready the craft for embarkation and launching in less than 5 minutes;

1.4	fully equipped;

1.5	to the extent that is practically possible, at a safe and protected place and protected against damage caused by fire and explosions. Survival craft on tankers shall not be stowed on or above a cargo tank, slop tank or other tank containing explosive or other hazardous cargo.

2. Lifeboats and life rafts with appurtenant launching appliances shall be stowed as close to rooms in the accommodation and work areas as possible.

3. Lifeboats that shall be lowered down along the side of the ship shall be stowed as far in front of the propeller as practically possible. When it is practically possible, the ship shall be arranged in such a way that the lifeboats in their stowed positions are protected against damage from rough seas.

4. Lifeboats must be stowed attached to launching appliances.

5. Each life raft shall be stowed with a painter that is permanently fastened to the ship.

6. Each life raft shall be stowed in a float-free appliance so that each of them will float free and, if they are inflatable, inflate automatically when the ship sinks.

7. Life rafts shall be stowed so that manual release of one raft or container at a time from the fastening appliances is possible.

8. Davit-launched life rafts must be stowed within reach of the lifting hooks unless some means of transfer is provided which is not rendered inoperable within the limits of trim and list specified in section 1.2 or by ship motion or power failure.

9. It must be possible to board and launch davit-launched life rafts from a place in the immediate vicinity of the place where they are stowed or from a place to which the life raft is moved prior to launching in accordance with section 7.

10. Embarkation appliances for survival craft shall be designed so that the lifeboats can be boarded and launched directly from the place where they are stowed.

# Annex 2 (cf. section 13)

# Muster stations

1. There shall be muster stations near the embarkation stations. Each muster station shall have sufficient space to hold all of the persons who are meant to muster at that station, with no less than 0.35 m² per person.

2. Muster and embarkation stations shall be readily accessible from accommodation and work areas.

3. Muster and embarkation stations above the sides of the ship shall be adequately illuminated by lighting supplied from the emergency source of electrical power, cf. the regulations in force at any given time concerning the construction of passenger and cargo ships.

4. Alleyways, stairways and exits giving access to the muster and embarkation stations shall be lighted. Such lighting shall be capable of being supplied by the emergency source of electrical power.

5. Davit-launched and free-fall launched survival craft muster and embarkation stations shall be so arranged as to enable stretcher cases to be placed in survival craft.

6. An embarkation ladder extending in a single length from the deck to the waterline in the ship’s lightest seagoing condition under unfavourable conditions of trim of up to 10° and a list of up to 20° either way shall be provided at each embarkation station or at every two adjacent embarkation stations for survival craft launched down the side of the ship.

7. Where necessary, means shall be provided for bringing the davit-launched survival craft against the ship's side and holding them alongside so that persons can be safely embarked.

# Annex 3 (cf. section 20)

# General alarm and loudspeaker system

1. The general alarm system

1.1	The general emergency alarm system should be capable of sounding the general alarm signal consisting of seven or more short blasts followed by one long blast on the vessel's whistle or siren and additionally on an electrically operated bell or klaxon or other equivalent warning system, which should be powered from the vessel's main supply and the emergency source of electrical power. The system shall be capable of operation from the navigating bridge. The system shall be audible throughout all the accommodation and normal crew working spaces. The alarm shall continue to sound after it has been triggered until it is turned off manually.

1.2	The minimum sound pressure levels for the emergency alarm tone in interior and exterior spaces must be 80 dB(A) and at least 10 dB(A) above ambient noise levels existing during normal equipment operation with the vessel underway in moderate weather. In cabins without a loudspeaker installation, an electronic alarm transducer, e.g. an acoustic alarm (a buzzer) or similar, is to be installed, cf. section 20, sub-section 5.

1.3	The sound pressure level at the sleeping position in cabins and in cabin bathrooms is to be at least 75 dB (A) and at least 10 dB (A) above ambient noise levels.

2.1 Public address system

2.1	The public address system is to be a loudspeaker installation that makes it possible to broadcast messages to all spaces where crew members or passengers are normally found and to muster stations. The system shall make it possible to broadcast messages from the navigating bridge. The public address system must be installed with regard to acoustically marginal conditions and is not to require any action from the addressee. It must be protected against unauthorised usage.

2.2	With the ship underway in normal conditions, the minimum sound pressure level for broadcasting emergency announcements is to be:

2.2.1	in interior spaces 75 dB (A) and at least 20 dB (A) above the speech interference level, and

2.2.2	in exterior spaces 80 dB (A) and at least 15 dB (A) above the speech interference level.

Annex 4 (cf. section 23)

Onboard training and instruction

1. Onboard training in the use of the vessel’s lifesaving appliances, including survival craft equipment, and in the use of the vessel’s fire-extinguishing appliances must be given as soon as possible but not later than 2 weeks after a crew member joins the vessel. If the crew member is on a regularly scheduled rotating assignment to the vessel, however, the training required in this section need be given only within 2 weeks of the time the crew member first joins the vessel. The crew must be instructed in the use of the vessel’s fire-extinguishing and lifesaving appliances and in survival at sea at the same interval as the drills. Individual units of instruction may cover different parts of the vessel’s lifesaving and fire-extinguishing appliances, but all the vessel’s lifesaving and fire-extinguishing appliances must be covered within any period of 2 months.

2. Every crew member must be given instructions that include, but are not necessarily limited to:

2.1	The operation and use of the vessel’s inflatable life rafts;

2.2	The problems of hypothermia, first aid treatment for hypothermia, and other appropriate first aid procedures;

2.3	Any special instructions necessary for use of the vessel’s lifesaving appliances in severe weather and severe sea conditions; and

2.4	The operation and use of fire-extinguishing appliances.

3. Onboard training in the use of davit-launched life rafts must take place at intervals of not more than 4 months on each vessel with davit-launched life rafts. Whenever practicable, this training must include the inflation and lowering of a life raft. If this life raft is a special life raft intended for training purposes only and is not part of the vessel’s lifesaving equipment, this life raft must be conspicuously marked.

# Annex 5 (cf. section 24)

# Drills

1. Each abandon ship drill shall include:

1.1	summoning of passengers and crew to muster stations by means of the general alarm;

1.2	reporting to stations and preparing for the duties described in the muster list;

1.3	checking that passengers and crew are suitably dressed;

1.4	checking that lifejackets are correctly donned;

1.5	lowering of at least one lifeboat after any necessary preparation for launching;

1.6	starting and operating the lifeboat engine;

1.7	operation of davits used for launching life rafts.

1.8	conducting a simulated search for and rescue of persons who are trapped in their cabins;

1.9	provision of instructions in the use of radio lifesaving appliances.

2. Different lifeboats must, as far as practicable, be lowered to comply with the requirements of sub-section 1.5 at successive drills.

3 Lowering into the water, rather than launching of a lifeboat arranged for free-fall launching, is acceptable when free-fall launching is impracticable, provided that the lifeboat is free-fall launched with its assigned operating crew aboard and is manoeuvred in the water at least once every 6 months. However, when compliance with the 6-month requirement is impracticable, the Norwegian Maritime Directorate may extend this period to 12 months, provided that arrangements are made for simulated launching at intervals of not more than 6 months

4. If lifeboat and rescue boat launching drills are carried out with the vessel making headway, such drills must, because of the dangers involved, be practiced in sheltered waters only and be under the supervision of an officer experienced in such drills.

5. Emergency lighting for mustering and abandonment must be tested at each abandon-ship drill.

6. Fire drills must, as far as practicable, be planned with due consideration given to regular practices in the various emergencies that may occur for that type of vessel and its cargo.

7. Each fire drill must include:

7.1	reporting to stations and preparing for the duties described in the muster list for the particular fire emergency being simulated;

7.2	starting of a fire pump and the use of at least two jets of water to determine that the system is in proper working order;

7.3	checking the firefighters’ equipment and other personal rescue equipment;

7.4	checking the relevant communications equipment;

7.5	checking the operation of watertight doors, fire doors, fire dampers and main inlets and outlets of ventilation systems in the drill area; and

7.6	checking the necessary arrangements for subsequent abandonment of the vessel.

8. The equipment used during drills must immediately be brought back to its fully operational condition. Any faults or defects discovered during the drills must be remedied as soon as possible.

# Annex 6 (cf. section 25)

# Operational readiness, maintenance and inspections

1. Operational readiness.

Before the vessel leaves port and at all times during the voyage, each lifesaving appliance must be in working order and ready for immediate use.

2. Instructions about maintenance, etc.

2.1	The manufacturer’s instructions for onboard maintenance of lifesaving appliances must be on board the vessel. Maintenance shall be carried out in accordance with these instructions.

2.2	Instructions for onboard maintenance of lifesaving appliances shall be easily understood, illustrated wherever possible, and, as appropriate, shall include the following for each appliance:

2.2.1	a checklist for use when carrying out the inspections required in section 7;

2.2.2	maintenance and repair instructions;

2.2.3	schedule of periodic maintenance;

2.2.4	diagram of lubrication points with the recommended lubricants;

2.2.5	list of replaceable parts;

2.2.6	list of sources of spare parts;

2.2.7	log for records of inspections and maintenance.

2.3	Instead of the instructions required in sub-sections 2.2.1-2.2.7, a maintenance programme can be utilised, which has been prepared for the ship and which includes the same requirements.

3. Maintenance of falls

3.1	Each fall used in a launching appliance on a vessel must be turned end for end at intervals of not more than 30 months and be renewed when necessary due to deterioration or at intervals of not more than five years, whichever is earlier.

3.2	Instead of the requirement in sub-section 3.1 that the falls be turned end for end, the Norwegian Maritime Directorate can approve periodic inspections of the falls and their replacement when necessary due to deterioration or at intervals of not more than four years, whichever is earlier.

4. Spare parts and repair equipment

Spare parts and repair equipment must be provided for each lifesaving appliance and component that either is subject to excessive wear or consumption or needs to be replaced regularly

6. Weekly inspections

The following tests and inspections must be carried out weekly:

6.1	Each survival craft, rescue boat, and launching appliance must be visually inspected to ensure its readiness for use;

6.2	all engines in lifeboats and rescue boats shall be run for a total period of not less than three minutes, provided the ambient temperature is above the minimum temperature required for starting and running the engine. During this period of time, it should be demonstrated that the gearbox and gearbox train are engaging satisfactorily. If the special characteristics of an outboard motor fitted to a rescue boat would not allow it to be run other than with its propeller submerged for a period of 3 minutes, it should be run for such period as prescribed in the manufacturer's handbook, and

6.3 the general emergency alarm system shall be tested.

7 Monthly inspections.

Inspections of the lifesaving appliances, including the lifeboat equipment must be conducted monthly, using the checklist that is required in sub-section 2.2 to make sure it is complete and in good order. A report of the inspection shall be entered in the log.

7182T

8 Servicing of inflatable life rafts, inflatable lifejackets and inflated rescue boats.

8.1	All inflatable life rafts, inflatable lifejackets and marine evacuation systems shall be serviced:

8.1.1 at no more than 12-month intervals, but in all cases where this is practically impossible, the Norwegian Maritime Directorate can extend the time interval to 17 months; and

8.1.2 at an approved servicing station that is competent to service them, maintains proper servicing facilities and uses only properly trained personnel.

8.2	If the Norwegian Maritime Directorate approves arrangements for inflatable life rafts that are new and that have new characteristics, it can permit an extension of the interval between servicing on the following terms:

8.2.1	Arrangements that stick to the same standard as that required by the procedure for testing during the extended interval between servicing.

8.2.2	The life raft system shall be inspected onboard by certified personnel in accordance with sub-section 8.1.1.

8.2.3	Servicing at intervals that do not exceed five years.

8.3	All repairs and all maintenance of inflated rescue boats shall be conducted in accordance with the manufacturer’s instructions. Emergency repairs can be conducted on board the ship. However, permanent repairs shall be conducted by an approved servicing facility.

9 Periodic servicing of hydrostatic release units

Hydrostatic release units, other than disposable hydrostatic release units, shall be serviced:

9.1	at intervals not exceeding 12 months, but in all cases where this is practically impossible, the Norwegian Maritime Directorate may extend this period to 17 months; and

9.2	at an approved servicing station that is competent to service them, maintains proper servicing facilities and uses only properly trained personnel.

10 Marking of stowage locations

Containers, brackets, racks, and other similar stowage locations for lifesaving equipment shall be marked with symbols indicating the devices stowed in that location for that purpose. If more than one lifesaving device is stowed in that location, the number of devices shall also be indicated.

11 Periodic servicing of launching appliances and on-load release gear

11.1 Launching appliances:

11.1.1	shall be serviced at recommended intervals in accordance with instructions for on-board maintenance as required in sub-section 2.2;

11.1.2	shall be subjected to a thorough examination at intervals not exceeding five years; and

11.1.3	shall, upon completion of the examination in section 11.1.2, be subjected to a dynamic test of the winch brake at a proof load at least 1.1 times the maximum working load of the winch at the maximum lowering speed.

11.2 Lifeboat on-load release gear shall be:

11.2.1	serviced at recommended intervals in accordance with instructions for on-board maintenance as required in sub-section 2.2;

11.2.2	subjected to a thorough examination and test by properly trained personnel who are familiar with the devices in connection with required inspections, cf. the regulations concerning the inspection of passenger ships and cargo ships that are in force at any given time; and

11.2.3	operationally tested under a load of 1.1 times the total mass of the lifeboat when loaded with its full complement of persons and equipment whenever the release gear is overhauled. Such overhauling and testing shall be carried out at least once every 5 years.

Information on the impact assessment

The draft regulations give minimum requirements on life saving appliances on cargo ships. Those cargo ships that today hold a minimum level according to existing regulations will probably experience the draft regulations as an increase in the requirements of life saving appliances on board. At the same time, the far most of today’s fleet lies above what is the minimum standard, and thus the impact is estimated to be less of a burden to the industry to meet the new requirements.

The economic impact on the industry will be relatively small. Many cargo ships are already equipped to meet the new requirements.

The draft regulations will have no impact on the Administration in form of administrative burdens. There will be no extra burden on the Administration regarding survey according to the new requirements.