Draft
Regulation concerning the carriage of cargoes on cargo ships

Laid down by the Norwegian Maritime Directorate (date…) on the legal basis of Act of 9 June 1903 no. 7 relating to Public Control of the Seaworthiness of Ships, etc., § 1, § 41a, § 42, § 68, § 69 and § 100, cf. Royal Decree of 12 October 1962, Royal Decree of 5 April 1963 and Royal Decree of 1 December 1978 and Formal Delegation of 15 January 1979 No. 1. Cf. the EEA Agreement Amendment II Chapt. XIX No. 1 (Directive 98/34/EC as amended by Directive 98/48/EC) and Act of 17 December 2004 No. 101.
Part 1
General provisions
§ 1
Scope of application

This Regulation applies to all Norwegian cargo ships and lighters/barges, unless otherwise provided for in the individual Sections.


This Regulation applies to the carriage of cargoes, except liquid substances in bulk, gas in bulk and aspects of the carriage which are covered by other regulations, which, owing to the special danger those aspects represent to the ship or those on board may require special precautions on all ships covered by this Regulation.
§ 2
Definitions

For the purposes of this Regulation:

(1)
BC Code means the Code of Safe Practice for Solid Bulk Cargoes, adopted by the IMO by resolution A.434(XI), as subsequently amended.

(2)
BLU Code means the Code of Practice for the Safe Loading and Unloading of Bulk Carriers, adopted by the IMO by resolution A.862(20), as subsequently amended. 

(3)
CSS Code means the Code of Safe Practice for Cargo Stowage and Securing, adopted by the IMO by resolution A.714(17), as subsequently amended.

(4)
Grain Code means the Code for the Safe Carriage of Grain in Bulk, adopted by the IMO by resolution MSC.23(59), as subsequently amended.


(5)
Cargo Securing Manual means a manual for the securing of cargo, as required by SOLAS, prepared pursuant to IMO guidelines (MSC/Circ.745).


(6)
MARPOL 73/78 means the International Convention for the Prevention of Pollution from Ships, 1973, as modified by the Protocol of 1978, as subsequently amended.


(7)
SOLAS means the International Convention for the Safety of Life at Sea 1974, as subsequently amended.

§ 3
Obligations of the company and the master

The company and the master shall ensure the compliance with the provisions of this Regulation.

§ 4
Exemptions


The Norwegian Maritime Directorate may, in individual cases and upon written application, grant exemption from the requirements of this Regulation. There must be special reasons that make the exemption necessary and it must be justifiable in terms of safety. Exemptions can only be granted where they do not contravene international agreements to which Norway has acceded.
§ 5
Relationship to international legislation

The following international legislation applies to ships covered by this Regulation, unless otherwise explicitly provided for in the individual provisions:
a) SOLAS, Chapter VI;
b) BC Code;

c) CSS Code, subchapter 1.9.;

d) Grain Code.


The international entry into force shall be complied with. In absence of an international entry into force date, the international adoption date shall be complied with. Another entry into force date may still be provided for.
Part 2

Special provisions

§ 6
Cargo information


The shipper shall provide the master or his representative with information on the cargo sufficiently in advance of loading to enable the precautions which may be necessary for proper stowage and safe carriage of cargo to be put into effect. Such information shall be confirmed in writing and by appropriate shipping documents prior to loading the cargo on the ship.

The cargo information shall include:


a)
in the case of general cargo, and of cargo carried in cargo units, a general description of the cargo, the gross mass of the cargo or of the cargo units, and any relevant special properties of the cargo;


b)
in the case of bulk cargo, information on the stowage factor of the cargo, the trimming procedures, likelihood of shifting including angle of repose, if applicable, and any other relevant special properties. In the case of a concentrate or other cargo which may liquefy, additional information in the form of a certificate on the moisture content of the cargo and its transportable moisture limit;

c)
in the case of bulk cargo not classified in accordance with the provisions of SOLAS Chapter VII, but which has chemical properties that may create a potential hazard, in addition to the information required by paragraph b), information on its chemical properties.
§ 7
Acceptability for shipment of bulk cargoes

Concentrates or other cargoes which may liquefy shall only be accepted for loading when the actual moisture content of the cargo is less than its transportable moisture limit.


Cargoes which may liquefy during carriage shall be subject to periodical controls every six months
.

Prior to loading a bulk cargo which is not a cargo classified in accordance with the provisions of SOLAS, Chapter VII, but which has chemical properties that may create a potential hazard, special precautions for its safe carriage shall be taken, cf. Section 5.
§ 83
Cargo Securing Manual, booklet for stability data and strength and loading and unloading plan4

An approved Cargo Securing Manual shall be kept on board. Cargo ships holding certificates for domestic trade and lighters/barges in all trade areas shall keep a Cargo Securing Manual on board, but there are no requirements for the approval of the Manual by the Norwegian Maritime Directorate, or whoever is authorized by the Directorate.

To enable the master to prevent excessive stresses in the ship’s structural members, the ship shall be provided with a booklet for stability data and strength written in English. This requirement does not apply to cargo ships holding certificates for domestic trade and lighters/barges in all trade areas.

Before a solid bulk cargo is loaded or unloaded, the master and the terminal representative shall agree on a loading and unloading plan which shall ensure that the permissible forces and moments on the ship are not exceeded during loading or unloading. This requirement does not apply to cargo ships holding certificates for domestic trade and lighters/barges in all trade areas.
§ 9
Documentation

Ships carrying cargoes covered by this Chapter shall, to the extent they are covered by the body of legislation, keep the following documentation on board:


a)
BC Code;


b)
BLU Code;


c)
CSS Code;

d)
Booklet for stability data and strength, cf. Section 8;


e)
Document of authorization for the carriage of grain, where the Grain Code so requires;

f)
Grain Code;

g)
Loading and unloading plan, cf. Section 8;


h)
Cargo Securing Manual, cf. Section 8;


i)
SOLAS, Chapters VI and VII.

Part 3

General provisions

§ 10
Penalty

Willful or negligent violation of this Regulation shall be punishable by fines pursuant to the General Civil Penal Code of 22 May 1902 no. 10, § 339 subsection 2, § 48a and 48b, unless a more severe penalty is applicable under another statutory provision.

§ 11
Entry into force

This Regulation enters into force on 1 January 2006.


As of the same date, the Regulations of 21 May 1987 No. 406 concerning carriage by ship of special or dangerous cargoes in bulk or as packaged goods, are repealed.
1 The information required by subchapter 1.9. of the CSS Code shall be provided.





�  Laboratory examinations pursuant to the BC Code will be carried out by SINTEF.


3 Refer to the Regulation of 29 August 2003 No. 1114 concerning the safe loading and unloading of bulk carriers.


4 BLU Code.





	H:\(eng) Forskr.utkast frakt av last på lasteskip2.doc  -  06.07.2005
	


	H:\(eng) Forskr.utkast frakt av last på lasteskip2.doc  SaveDate: 04.07.2005 15:14
	4



